Real-Time Embedded Vision and Multitasking System on FPGA

ECE3073 Mini-Project Report

Group Number: 1

Student Names and IDs

| No. | Full Name | Student ID |
| --- | --- | --- |
| 1 | You Qing Liew | 33590400 |
| 2 | Melvin | 33566380 |
| 3 | Zhi Hang | 33524513 |
| 4 | Kenneth Lai | 32946562 |

Abstract

In this project, we built a system on the DE10-Lite FPGA board. The system can see numbers and show them in many ways. We used two NIOS II soft CPUs inside the FPGA. We also used a Grove Vision AI V2 camera and a Xiao ESP32-C3 board.

The camera sees a number (like a water meter reading). The ESP32 sends the number to the FPGA over SPI. The FPGA saves the number in shared memory. Then it shows the number on the VGA screen, the 6 HEX displays, the red, yellow, and green LEDs, and the on-board buzzer (as Morse code).

The two CPUs work at the same time. One CPU handles the camera and screen. The other CPU plays the message in Morse code. We also made two small Verilog blocks to help us measure how fast the system works.

I. INTRODUCTION

This project is for the ECE3073 unit. The goal is to build a real-time vision system on an FPGA. We used the DE10-Lite board because it has a MAX 10 FPGA chip that supports NIOS II soft CPUs.

We also used the Grove Vision AI V2 camera. This is a small camera that can run an AI model by itself. In our demo, the camera reads digits from a water meter image. The number can be "02012", for example.

The camera is wired to a Xiao ESP32-C3 board. The ESP32 talks to the camera and then talks to the FPGA over SPI. The FPGA does NOT need to run the AI. It just gets the digits and shows them.

The final system shows the result in many ways at the same time. The user can press KEY0 and KEY1, and flip the SW switches on the DE10-Lite to change what happens. We also added two GPIO pins so we can measure the latency of the system in real hardware.

[Figure 1 (Mandatory): Block diagram of the system. Show Grove Vision AI V2 -> ESP32-C3 -> DE10-Lite (NIOS Core 1 + NIOS Core 2 + shared SDRAM + VGA + HEX + LEDs + buzzer + accelerometer). INSERT IMAGE HERE.]

II. METHODOLOGY AND SYSTEM IMPLEMENTATION

A. Hardware Platform

We used:

DE10-Lite FPGA board (MAX 10 chip 10M50DAF484C7G).

Quartus Prime 18.1 Lite Edition to build the hardware.

Platform Designer (Qsys) to make the NIOS II system.

Grove Vision AI V2 camera.

Xiao ESP32-C3 as a small bridge between the camera and the FPGA.

A normal VGA monitor.

The Quartus project is nios_led_test.qpf. The top Verilog file is nios_led_test.v. It connects the NIOS system to the LEDs, the accelerometer pins, the keys, and the GPIO header.

We made several Qsys systems during the milestones:

M1/MileStone1.qsys and M1/nios_system_Dram.qsys - first version with one NIOS, an SDRAM controller, and a basic VGA.

M2/dual_processor.qsys - final version with TWO NIOS II cores, the SDRAM controller, an Avalon mutex, the SPI master, the accelerometer SPI, the pixel-buffer DMA, the character-buffer DMA, lots of PIO blocks, and our custom us_counter block.

B. Two NIOS II CPUs (Task Splitting)

To do many things at the same time, we used two NIOS II cores. Each core does its own job.

Core 2 - Camera and Screen (code: M2/software/core2V3/core2.c, plus accelerometer.c, hex.c, sdram.c). This core does:

Set up the SPI interrupt (spi_isr).

Read the camera message over SPI.

Save the message in shared SDRAM.

Read the accelerometer (ADXL345) and find pitch, roll, and tilt.

Show numbers on the 6 HEX displays.

Draw on the VGA screen.

Read the KEY0 and KEY1 buttons.

Core 1 - Buzzer and Morse (code: M2/software/core1/core1.c). This core does:

Read the message from shared SDRAM.

Play the message in Morse code on the buzzer.

Turn on the red LED for a dot.

Turn on the yellow LED for a dash.

Show the switch state on the LEDs.

C. How the Two CPUs Share Data

The two CPUs share data through SDRAM memory.

Shared memory base address: 0x01000000.

A flag area at Shared_Sdram_Flag.

Buffer 1 at SDRAM_1 = 0x01100000.

Buffer 2 at SDRAM_2 = 0x01200000.

To stop both CPUs writing at the same time, we use an Avalon mutex (/dev/SHARED_SDRAM). Each CPU does:

Lock the mutex.

Read or write the SDRAM.

Unlock the mutex.

After a write, we call alt_dcache_flush(). This makes sure the other CPU reads the new data and not an old cached copy.

D. Grove AI V2 Integration

The Grove camera is connected to the ESP32-C3. The ESP32 runs the Seeed SSCMA library. Our code is in M2/SCROLLING_NUMBERS_WORKS_copy_*.ino.

The ESP32 does this in a FreeRTOS task:

Call AI.invoke() to run the AI model.

Throw away weak detections (score under 50).

Sort the digit boxes from left to right.

Build the digits into a string, like 02012\n.

Wait for the FPGA to read the string over SPI.

We also have two test sketches:

ESP32/spi/spi.ino - simple SPI test.

ESP32/UART_2/UART_2.ino - UART test at 9600 baud (used in early testing).

E. SPI Communication

The FPGA is the SPI master. The ESP32 is the SPI slave. The FPGA reads one byte at a time.

We use 4 register pointers in software:

SPI_RxData - the byte we got.

SPI_TxData - the byte we want to send.

SPI_Status - tells us when a byte is ready (RRDY bit).

SPI_Control - turns on the SPI interrupt (bit 0x80).

The chip select (SPI_SS) is a PIO pin. We pull it low, send one byte, then pull it high again.

To make the messages reliable, the ESP32 wraps each message in '<' and '>'. For example: <02012>. The FPGA waits for '<' before saving any bytes, and stops when it sees '>'.

F. SPI Interrupt Handler

The SPI interrupt is the most important part. We keep it VERY short:

Read one byte.

If it is '<', start saving.

If it is '>' or '\0', set a flag spi_message_ready = 1.

Otherwise, save the byte into spi_message[] and ask for the next byte.

We do NOT call printf, write to SDRAM, or beep inside the interrupt. All slow work is done in the main while(1) loop.

G. VGA Output

The VGA controller is written in Verilog (M1/vgacontroller.v).

VGA timing: 640x480 @ 60 Hz.

Pixel clock: 25 MHz, made from the 50 MHz clock by a PLL (VGAdriver.v).

Inside the FPGA we use a 320x240 frame buffer in dual-port RAM (pixel.v).

The controller halves the X and Y address so the picture fills the full screen.

We can draw in two ways:

Direct pixel write with write_pixel(addr, color) from vga_custom.c.

UP DMA drivers - alt_up_pixel_buffer_dma_* to draw pixels, lines, and boxes, and alt_up_char_buffer_* to draw text. We use double-buffering with alt_up_pixel_buffer_dma_swap_buffers() so the picture does not flicker.

H. HEX Displays, LEDs, Speaker, Accelerometer

HEX displays: Six 7-segment displays, controlled by two packed PIO ports (HEX012_BASE and HEX345_BASE). hex.c has helper functions like char_to_7seg(), hex_refresh(), hex_scroll_string() and hex_show_3_signed_values().

LEDs: Three PIO bits for red, yellow, and green. Red = Morse dot. Yellow = Morse dash. Green = 'message received OK'.

Buzzer: One PIO bit at BUZZER_BASE = 0x02021030. We toggle this bit fast in software to make a tone. The dot and dash lengths follow the standard 1 : 3 : 7 Morse ratios.

Accelerometer (ADXL345): Connected on the dedicated GSENSOR_* SPI pins. We open it with alt_up_accelerometer_spi_open_dev(). We read X, Y, Z and use atan2() to get pitch and roll. We also pick one of 6 labels (UP, DOWN, LEFT, RIGHT, FWD, BACK) based on which axis is biggest.

I. Scheduling: Polling vs Interrupts

We did NOT use a full RTOS like uC/OS-II. Instead we used a simple style:

Interrupts are used only for the SPI receive path. This is the most time-critical part.

Polling is used for the keys, switches, and the main while(1) loop. We use edge detection on the keys (compare prev and curr) and a small software debounce.

Two cores running at the same time act like a simple parallel scheduler. Core 2 always does I/O. Core 1 waits for a flag and then plays Morse.

J. Custom Verilog Blocks

We wrote two small Verilog files to help the system.

1) us_counter.v - a microsecond timer.

A 32-bit counter that ticks at 1 MHz.

Made by dividing the 50 MHz clock by 50.

Mapped at US_COUNTER_BASE = 0x02021070.

Used in core2.c to time blocks of code (start = IORD(timer, 0); ... end = IORD(timer, 0); printf(...)).

2) latency_counter.v - a 12-bit latency counter.

Counts at 50 MHz (each count = 20 ns).

Starts when a leader_signal goes high.

Stops when a follower_signal goes high.

Used for the latency measurement (next section).

K. Latency Measurement With GPIO

To measure how fast the NIOS reacts, we use two GPIO pins:

GPIO[2] = leader. Driven by a free-running 20-bit counter inside the FPGA. Also connected to a PIO that the NIOS reads.

GPIO[10] = follower. The NIOS software copies the PIO input and writes it back out to GPIO[10].

When GPIO[2] goes high, the latency counter starts. When GPIO[10] goes high, the counter stops. The final count is the round-trip latency in 20 ns steps. The number is shown on the HEX displays so we can read it directly.

[Figure 2 (Mandatory): Platform Designer view of M2/dual_processor.qsys (showing two NIOS cores, SDRAM, mutex, SPI master, pixel-buffer DMA, PIOs, us_counter, latency_counter). Also include a photo of the DE10-Lite + Grove AI V2 + ESP32 set-up. INSERT IMAGE HERE.]

III. MAIN CONTROL ALGORITHM

The pseudocode below shows what each CPU does in its main loop. It is based on core2.c and core1.c.

Algorithm 1: Dual-Core Vision + Multitasking Loop

CORE 2 (Camera / Screen):

set up PIOs, HEX, VGA

open accelerometer

spi_interrupt_init()                # register spi_isr

open mutex /dev/SHARED_SDRAM

enable NIOS interrupts

prev_key0 = 1

prev_key1 = 1

while True:

# 1. draw on VGA

clear back buffer

draw pixels / lines / boxes

swap_buffers()

# 2. read shared switch + keys

lock(mutex)

swBase = read flag from SDRAM

unlock(mutex)

curr_key0 = KEY & 0x1

curr_key1 = KEY & 0x2

# 3. accelerometer mode or blank HEX

if (swBase & 0x1):

read pitch, roll, z

show them on HEX

else:

blank HEX

# 4. KEY0 pressed -> start SPI receive

if prev_key0 == 1 and curr_key0 == 0:

debounce()

spi_begin_receive_sentence()

# 5. ISR needs next byte

if spi_receiving and spi_next_byte_needed:

wait ESP32_QUEUE_DELAY

spi_start_one_byte_transfer(0xFF)

# 6. full message ready

if spi_message_ready:

store message in SDRAM

if spi_overflow:

reset SPI

# 7. KEY1 pressed -> play back

if prev_key1 == 1 and curr_key1 == 0:

debounce()

print_sentence_from_sdram()

process_camera_data()

prev_key0 = curr_key0

prev_key1 = curr_key1

ISR  spi_isr():

while SPI_Status & RRDY:

rx = SPI_RxData & 0xFF

if rx == '<':           start storing

elif rx == '>' or '\0': set spi_message_ready = 1

else:                   save rx, ask for next byte

CORE 1 (Morse / Buzzer):

open same mutex

read message from SDRAM

play_morse_string(message)   # buzzer + red/yellow LEDs

loop forever

IV. INNOVATION AND OPTIMISATION

Things we added on top of the basic requirements:

'<' and '>' markers in the SPI messages. This makes the system sync correctly even if it starts in the middle of a message.

Short SPI ISR. No printf, no SDRAM write, no beep inside the interrupt. All slow work runs in the main loop.

us_counter.v - a custom 1 MHz hardware counter. We use it to time our code very accurately.

latency_counter.v - measures system latency in real hardware with GPIO pins.

Dual-core SDRAM matrix kernel. Core 1 runs a small matrix-multiply against the shared SDRAM, with mutex and cache flushing. This proves the two CPUs work in parallel.

Smart HEX display. The decimal point on each HEX is used to show the sign (+ or -) of the accelerometer value.

Morse with sound + light. Red LED for dot, yellow LED for dash, green LED + short beep for 'message received'.

Smooth VGA drawing. RGB565 camera pixels are upscaled to 30-bit RGB. We use double-buffering so the screen does not flicker.

V. RESULTS AND DISCUSSION

We tested the full system on the DE10-Lite board.

What works:

The Grove camera reads the 5-digit water meter number.

The number goes through the ESP32 -> SPI -> FPGA and is saved into SDRAM.

The HEX, LEDs, buzzer and VGA all work at the same time.

KEY0 starts a new SPI receive.

KEY1 plays back the stored message and refreshes the camera frame on the VGA.

In accelerometer mode (SW1 = 1), tilting the board updates the HEX in real time.

Core 1's matrix kernel finishes correctly using only the mutex and shared SDRAM.

The system did not crash during repeated button presses.

Problems we hit:

The ESP32 needs a small wait between bytes (ESP32_QUEUE_DELAY). Without it, the first byte after '<' was sometimes missed.

The Avalon SPI core only has one interrupt (RRDY). So we had to toggle the chip select in software.

The two CPUs need alt_dcache_flush() after every shared write. Otherwise Core 1 sees old data.

The UART path at 9600 baud is slower than SPI. We used UART only for early debugging.

[Figure 3 (Mandatory): Final demo result. Show (a) the VGA screen with the digits, (b) the HEX displays in accelerometer mode, (c) an oscilloscope or logic analyser capture of GPIO[2] and GPIO[10] for the latency measurement. INSERT IMAGE HERE.]

VI. CONCLUSION

We built a complete real-time vision and multitasking system on the DE10-Lite FPGA. The system uses:

A Grove Vision AI V2 camera (through a Xiao ESP32-C3 SPI bridge).

Two NIOS II soft CPUs.

Shared SDRAM with an Avalon mutex.

Many outputs: VGA, HEX, LEDs, buzzer, accelerometer.

Two custom Verilog blocks (us_counter and latency_counter).

We learned how to:

Build hardware in Quartus and Platform Designer.

Write Verilog for custom peripherals.

Set up a dual-core NIOS II software project.

Use SPI and UART communication.

Write a clean interrupt handler.

Share memory between two CPUs.

Measure performance directly on the FPGA.

REFERENCES

[1] Seeed Studio, Grove Vision AI V2 - Wiki & Datasheet, https://wiki.seeedstudio.com/grove_vision_ai_v2/

[2] Intel Corporation, Nios II Processor Reference Guide, NII-PRG, 2020.

[3] Intel Corporation, Nios II Software Developer's Handbook, NII-SDH, 2020.

[4] Terasic Inc., DE10-Lite User Manual, Rev. 2.1.

[5] Analog Devices, ADXL345 Data Sheet, Rev. G.

[6] Intel Corporation, Embedded Peripherals IP User Guide (Avalon Mutex, Avalon SPI, Avalon PIO), UG-01085.

[7] Altera University Program, Video IP Cores (altera_up_avalon_video_pixel_buffer_dma, altera_up_avalon_video_character_buffer_with_dma, altera_up_avalon_accelerometer_spi).

[8] Seeed Studio, Seeed_Arduino_SSCMA GitHub repository.

[9] Espressif Systems, ESP32-C3 Technical Reference Manual and ESP32SPISlave Arduino library.